OVERSIZE FUEL TANK SENDING UNIT/PICKUP ASSEMBLY

Part Numbers: 1101                                                                                Sender Specifications:  73-10 ohm

Applications:

1971-1973 Ford Mustang

General Info: This sending unit assembly features oversize ½” tubes for high horsepower applications. The ½” return tube is ideal when using a return style regulator. The sending unit works with stock fuel level gauges. Compatible with gasoline or alcohol fuels.

Pickup Tube Size: ½”                    Feed and Return Tube:  ½” barb (or -8AN/-10AN with option PN 1070/1080)

Installation: Installation is very similar to installing a stock type sending unit (see repair manual).

O-Ring: The RobbMc assembly uses an off-the-shelf #229 o-ring (included) to seal to the tank. While not the same as the stock o-ring, replacement o-rings are available at most hardware and auto parts stores.

Wiring: The factory wire connector will not attach to the RobbMc unit. Connect the red wire to the factory wire that connects to the fuel level gauge. Connect the black wire to ground.

Filter:  This RobbMc assembly is designed to be used without the factory style in-tank “sock” filter as these filters become restrictive over time and are difficult to service. Use an external inline prefilter such as RobbMc PN 1025 or 1029.

Return Line: The return tube is ½” to allow the use of a ½” or -8AN return line from a return style regulator. If using a mechanical pump with a ¼” “vapor” return line, connect a short section of ½” rubber line to the tube and then reduce it to ¼” with a reducer fitting (available from RobbMc as PN 1071). If no return line is used, cap off the return tube.

Notes:

~ This sending unit will not operate the optional factory low fuel level warning light.

~ When racing, try to keep the gas tank at least half full to keep the fuel from sloshing away from the pickup tube and sucking air into the tube.

~ Due to variations in tanks and sending units, the fuel level gauge may read slightly higher or lower than with the stock sending unit.

~When connecting braided AN lines to the tubes, be sure to hold the RobbMc fitting with one wrench while tightening the AN line with another wrench to avoid damage to the aluminum tube.

~If the gauge doesn’t work after installation, make sure the black wire has a good ground and make sure the red wire is connected to the factory wire that runs up to the gauge.

~For reference, the feed tube is on the passenger side and the return tube is on the driver’s side.