OVERSIZE FUEL TANK SENDING UNIT/PICKUP ASSEMBLY

Part Number: 1103                                            Sending Unit Specifications:  0-90 ohm

Applications: 1973-1978 Toronado
1973-1976 Impala, Caprice, Electra, LeSabre, Riviera, 98, Delta 88, Catalina, Bonneville
1973 Centurion
1973-1975 Bel Air

General Info: This unit features oversize ½” tubes for high horsepower applications. The ½” return tube is ideal when using a return style regulator. Works with stock fuel level gauges. Compatible with gasoline or alcohol fuels.

Pickup Tube Size: ½”                      Feed and Return Tube:  ½” barb (or -8AN/-10AN with option PN 1070/1080)

EEC/Vent Tube Size: 5/16” (same as OEM). The factory EEC/vent hose can be reused but may require trimming.

O-Ring: The RobbMc assembly uses an off-the-shelf #339 o-ring (included) to seal to the tank. While not the same as the stock o-ring, replacement o-rings are available at most hardware and auto parts stores.

Wiring: The black wire must be connected to ground in the same manner as the factory sending unit. The factory fuel gauge wire must be connected to the red wire.

Filter:  This RobbMc assembly is designed to be used without the factory style in-tank “sock” filter. Use an external inline prefilter such as RobbMc PN 1025 or 1029.

Return Line: The return tube is ½” to allow the use of a return line from a return style regulator. If using a mechanical pump with a ¼” “vapor” return line, connect a short section of ½” rubber line to the tube and then reduce it to ¼” with a reducer fitting (available from RobbMc as PN 1071). If no return line is used, cap off the return tube.

Notes:

~ When racing, try to keep the gas tank at least half full to keep the fuel from sloshing away from the pickup tube.

~ Due to variations in tanks/sending units, the fuel level gauge may read slightly different than with the OEM unit.

~When connecting AN lines to the tubes, hold the RobbMc fitting with one wrench while tightening the AN line with another wrench to avoid damage to the aluminum tube.

~If the gauge doesn’t work after installation, make sure the black wire has a good ground.