                                                              [image: image3.jpg]



        OVERSIZE FUEL TANK SENDING UNIT/PICKUP ASSEMBLY
Part Number: 1088                                            Sending Unit Specifications:  0-90 ohm 

Applications: 1978-1988 GM "G-body" cars (except station wagon) including Century, Regal, Malibu, Monte Carlo, Cutlass, Bonneville, Grand Am, Grand Prix, and Lemans.

Note: Unit will fit tanks from fuel injected cars but is not designed to be used with an in-tank pump. 
General Info: This unit features oversize ½” tubes for high horsepower applications. The ½” return tube is ideal when using a return style regulator. Works with stock fuel level gauges. Compatible with gasoline or alcohol fuels.
Pickup Tube Size: ½”                      Feed and Return Tube:  ½” barb (or -8AN/-10AN with option PN 1070/1080)
EEC/Vent Tube Size: 5/16” (same as OEM). The factory EEC/vent hose can be reused but may require trimming.
O-Ring: The RobbMc assembly uses an off-the-shelf #339 o-ring (included) to seal to the tank. While not the same as the stock o-ring, replacement o-rings are available at most hardware and auto parts stores.
Wiring: The black wire must be connected to ground in the same manner as the factory sending unit. The factory fuel gauge wire must be connected to the red wire. 
Filter:  This RobbMc assembly is designed to be used without the factory style in-tank “sock” filter. Use an external inline prefilter such as RobbMc PN 1025 or 1029.
Return Line: The return tube is ½” to allow the use of a return line from a return style regulator. If using a mechanical pump with a ¼” “vapor” return line, connect a short section of ½” rubber line to the tube and then reduce it to ¼” with a reducer fitting (available from RobbMc as PN 1071). If no return line is used, cap off the return tube.
Notes:

~ When racing, try to keep the gas tank at least half full to keep the fuel from sloshing away from the pickup tube. 
~ Due to variations in tanks/sending units, the fuel level gauge may read slightly different than with the OEM unit.

~When connecting AN lines to the tubes, hold the RobbMc fitting with one wrench while tightening the AN line with another wrench to avoid damage to the aluminum tube. 45 degree tube ends recommended (see photo below).
~If the gauge doesn’t work after installation, make sure the black wire has a good ground. 

[image: image1.jpg]mPerformance ™




[image: image2.jpg]









